# Reproducible O-RU Power Measurement (Software Proxy)

TEEP Final Report Draft (Markdown-first)

Jesaya David

2026-02-05

# Abstract

This work develops a reproducible power-measurement methodology for an O-RAN Radio Unit (O-RU), with an initial software-first proxy executed on a general-purpose Linux host. Because RU hardware and lab-grade instrumentation were not always available, we measure platform power using Intel/AMD RAPL energy counters exposed via Linux and collected by Scaphandre.

We define repeatable workload states (Idle, Active-Idle, Load-L/M/H), synchronize scenario boundaries via explicit UTC markers, and compute power/energy statistics on trimmed steady-state windows. A multi-round sweep shows a strong Idle→Active step and a narrower active band across load levels. A “burst vs smooth” proxy experiment demonstrates that race-to-sleep (TDM-style duty cycling) can reduce average platform power by ~48% at iso-throughput, motivating scheduler-level future work.

Scope note: Reported power values are platform power under RU-like workload, not RU AC/DC input power.

# 1. Background and Goal

## 1.1 Problem statement

Energy consumption in RAN infrastructure is dominated by the base station and radio unit. To compare energy behavior across configurations and ultimately across vendors/hardware, we need a measurement procedure that is: - repeatable (same scenario → same result within noise), - traceable (clear measurement point, units, timestamps), - reviewer-safe (explicit windowing, clear limitations).

## 1.2 Project goal

Establish a reproducible methodology that can be executed now (software proxy) and later transferred to hardware-grade RU input power measurement without changing the scenario structure.

# 2. Measurement Point and Metrics

## 2.1 Measurement point (software proxy)

- Device under test (DUT): General-purpose compute host running RU-like workload proxy.

- Power instrument: Scaphandre (Prometheus exporter) reading Linux RAPL counters.

- Signal: scaph_host_power_microwatts scraped at ~10 s cadence.

## 2.2 Metrics

We compute per-state metrics on steady-state windows: - Average power: Pavg (W) - Energy: E (J) - Throughput: T (Gbps) - Efficiency (proxy): η=T/Pavg (Gbps/W)

Energy is approximated from samples as:

E≈i​PiΔt

# 3. Scenario Definition

## 3.1 States

- Idle: OS baseline, no traffic.

- Active-Idle: measurement/stack active, traffic off.

- Load-L/M/H: offered load steps (nominally 30% / 60% / 100%).

## 3.2 Run structure

To avoid boundary transients, we: - record explicit Start_* / Stop_* markers in UTC, - trim 10 s from the start and end of each marked segment, - score only the trimmed windows.

# 4. Tooling and Data Formats

## 4.1 Toolchain

- Scaphandre (power estimator)

- iperf3 (traffic generation)

- Bash scripts for orchestration

- Python scripts for parsing/plotting

## 4.2 Artifact conventions

- Raw runs live under runs/YYYY-MM-DD/...

- Plots and summaries live under assets/YYYY-MM-DD/plots/

# 5. Experimental Results

## 5.1 Pilot: Power-enabled pipeline validation (2026-01-20)

A pilot run validated end-to-end logging, markers, and per-state scoring.

Per-state summary (pilot)

| State | Mean Power (W) | Energy (J) | Duration (s) | Throughput (Gbps) | Efficiency (Gbps/W) |
| --- | --- | --- | --- | --- | --- |
| Pure Idle | 1.023 | 91.6 | 100 |  |  |
| Active-Idle | 1.859 | 319.7 | 181 |  |  |
| Load-M | 37.631 | 10704.6 | 292 | 97.464 | 2.590 |

Source artifacts: runs/2026-01-20/.

## 5.2 Week 3: Load sweep statistics (2026-01-22)

A multi-state sweep (Idle + Load-L/M/H) demonstrates repeatability and the “step-function” behavior:

| State | Mean (W) | Std (W) | CV (%) |
| --- | --- | --- | --- |
| Idle | 1.4920 | 0.7964 | 53.38 |
| Load-L | 50.5983 | 2.9533 | 5.84 |
| Load-M | 50.2996 | 3.4350 | 6.83 |
| Load-H | 48.8443 | 8.3225 | 17.04 |

Key observation: once traffic begins, platform power jumps from ~1–2 W to ~50 W, with only modest separation between L/M/H compared to the Idle→Active jump.

Supporting plots: - Timeline:  - Linearity:

## 5.3 Week 4: Gap analysis vs RU anchor classes (2026-01-28)

We produced a publishable sensitivity plot and a gap overlay comparing the proxy power trace to modeled RU classes.

Key trimmed-window medians (proxy): - Idle median: 5.696 W - Active median: 24.983 W - 30% load median: 23.353 W - 60% load median: 24.983 W - 100% load median: 26.926 W

RU anchors used (P12-style classes): - Micro RU: Pmin=59 W, Pmax=110 W - Macro RU: Pmin=197 W, Pmax=531 W

Derived ratios (RU / proxy): - 59/5.696≈10.36× (micro idle vs proxy idle) - 197/5.696≈34.59× (macro idle vs proxy idle)

Figures: - Sensitivity:  - Gap overlay:

## 5.4 Two-host sweep (Windows sink) (2026-02-05)

To increase credibility beyond localhost/loopback, we repeated the sweep with two hosts: - Measured host (Linux laptop): power logging + iperf3 client - Sink (Windows PC): iperf3 server

Artifacts: - Run folder: runs/2026-02-05/sweep-120233/ - Stats table: assets/2026-02-05/plots/stats_summary.md - Repeatability table: assets/2026-02-05/plots/repeatability_per_run.md - Throughput table (client logs): runs/2026-02-05/sweep-120233/iperf_client_summary.md

Trimmed-window power summary:

| State | Mean (W) | Std (W) | CV (%) | Min (W) | Max (W) |
| --- | --- | --- | --- | --- | --- |
| Idle | 3.4075 | 0.2798 | 8.21 | 2.9361 | 4.1295 |
| Load-L | 6.1682 | 0.3811 | 6.18 | 5.0077 | 6.8600 |
| Load-M | 6.1960 | 0.3657 | 5.90 | 5.3906 | 7.3279 |
| Load-H | 6.5841 | 1.4690 | 22.31 | 5.3178 | 16.3342 |

Notes: - This run used TCP mode (best-effort). The L/M/H labels are nominal; achieved throughput depends on link capacity and TCP behavior. - Offered throughput was Wi‑Fi-limited in this run; achieved receiver rates were ~51–61 Mbps across L/M/H. - Load-H shows a high CV due to a single large spike (max 16.33 W) concentrated in Run 2; repeatability per-run statistics isolate this event.

Supporting plots: - Timeline:  - Linearity:

## 5.5 Validation: Burst vs smooth at iso-throughput (2026-02-04)

We tested a “race-to-sleep” proxy using duty-cycled traffic.

| Phase | Duration (s) | Avg Power (W) |
| --- | --- | --- |
| Idle baseline | 60.0 | 4.79 |
| Smooth (30%) | 180.0 | 21.78 |
| Burst (30% duty) | 180.2 | 11.25 |

Savings: 21.78−11.25=10.53 W (48.4%).

# 6. Discussion

## 6.1 What the proxy can and cannot claim

This methodology reliably measures platform power response to workload state changes. It does not directly measure: - RU power amplifier behavior, - RU AC/DC input power, - RF output power or EIRP.

## 6.2 Interpreting step-function behavior

The Week 3/4 sweeps suggest the platform enters a high-power operating regime as soon as sustained traffic begins (CPU frequency/uncore/NIC activity), after which incremental throughput changes do not strongly increase platform power.

## 6.3 Relation to prior work (context for the “gap”)

Our observations are consistent with the recurring theme in base-station power modeling literature: static/offset power dominates at low load, so efficiency gains require enabling real sleep states rather than only reducing utilization.

For an annotated literature bridge (working notes), see: - “How Much Energy is Needed to Run a Wireless Network?” (E³F framework; static power issue) - “Energy Efficiency Improvements through Micro Sites …” (offset power importance) - “A Parameterized Base Station Power Model” (load/bandwidth/antenna parameterization) - “Deploying Dense Networks for Maximal Energy Efficiency …” (circuit power dominance; multiplexing)

# 7. Limitations and Threats to Validity

- Software proxy only (no RU AC/DC input).

- Sampling cadence (~10 s) limits transient analysis.

- Loopback/localhost traffic can overemphasize CPU/network stack vs NIC/PHY behavior.

- Background OS tasks and CPU governor can bias the “active band.”

# 8. Data Quality and Acceptance Criteria (Proposed)

These criteria are chosen to be practical for an internship report while still being reviewer-safe.

## 8.1 Repeatability (power)

For each state (Idle, Active-Idle, Load-L/M/H), compute the coefficient of variation:

CV=σμ×100%

Proposed thresholds (steady-state, trimmed windows): - Load states (L/M/H): target CV≤10% (good), CV≤5% (strong). - Active-Idle: target CV≤10%. - Idle: may exhibit higher CV due to low absolute power; accept if Idle is clearly separated from Active band and the windowing policy is consistent.

## 8.2 State separability (sanity)

- Ordering check (typical): Pidle<Pactive<Pload.

- Practical check: state-to-state differences should be visually obvious in the timeline plot and supported by per-state statistics (mean/median).

# 9. Reproducibility (How to Run)

## 9.1 Prerequisites (Ubuntu / native Linux on the measured host)

- RAPL must be available (typically /sys/class/powercap).

- Scaphandre Prometheus exporter running and reachable at http://localhost:8080/metrics.

- iperf3 installed.

Start Scaphandre (Docker example):

sudo docker run --rm --privileged --pid=host \
    -p 8080:8080 \
    -v /proc:/proc:ro \
    -v /sys:/sys:ro \
    hubblo/scaphandre prometheus --address 0.0.0.0 --port 8080

Start iperf3 server (same host for loopback experiments):

iperf3 -s

## 9.2 Two-host topology (recommended for credibility)

Use a separate sink host to avoid loopback artifacts.

- Host A (Measured DUT, Ubuntu): runs Scaphandre + the iperf3 client + the sweep scripts.

- Host B (Traffic sink, Windows/WSL acceptable): runs an iperf3 server.

Notes for Windows/WSL sink: - Simplest: run iperf3 server natively on Windows (better throughput, fewer networking surprises). - If you run the server inside WSL2, inbound connections may require Windows port forwarding due to WSL2 NAT.

## 9.3 Week 4 sweep (gap analysis inputs)

This produces a new run folder containing power_uw.txt, markers.csv, and per-segment iperf logs.

./scripts/run_week4_gap_run.sh

Optional overrides (choose targets that match your link; e.g., 1 GbE cannot sustain 30G):

TARGET_HOST=127.0.0.1 MODE=udp TARGET_L=30G TARGET_M=60G TARGET_H=90G \
OUTPUT_DIR=runs/$(date -u +%Y-%m-%d)/sweep-01 \
./scripts/run_week4_gap_run.sh

Analyze and generate plots/stat summaries:

python3 scripts/analyze_week3_data.py runs/2026-01-28/sweep-01

## 9.4 Burst experiment

Run:

./scripts/run_burst_experiment.sh

Notes: - The run artifacts go under runs/YYYY-MM-DD/burst-experiment/. - The analysis script currently reads power_uw.txt from the repository root (not from the run folder). If your power log is stored elsewhere, copy/symlink it to ./power_uw.txt before running the analysis.

Analyze:

python3 scripts/analyze_burst_experiment.py

## 9.5 Export to PDF (optional)

If you have pandoc installed:

pandoc docs/FinalReport/00_Final_Report_Draft.md -o final_report.pdf

# 9. Conclusions

- A marker-aligned, trimmed-window procedure yields stable, reviewer-safe per-state power statistics.

- The proxy platform shows a strong Idle→Active jump and a narrower active band across L/M/H.

- Duty-cycled “bursting” reduces average platform power substantially at iso-throughput, motivating scheduler-level TDM approaches.

# 10. Future Work

- Hardware-in-loop measurement at RU input power (PDU/analyzer), preserving the same scenario states and marker policy.

- Multi-host (real NIC path) validation to reduce loopback artifacts.

- Scheduler modifications to create controllable idle gaps (TDM) and verify micro-sleep entry on RU hardware.

# References (draft)

## Internal (repo)

- Methodology, SOP, and results drafts under docs/FinalReport/.

- Study notes and experiment logs under docs/StudyNotes/.

- Raw artifacts under runs/ and derived plots under assets/.

## Literature notes (need full BibTeX later)

- assets/2026-02-02/Paper1.md

- assets/2026-02-02/Paper2.md

- assets/2026-02-02/Paper3.md

- assets/2026-02-02/Paper4.md